# Economic Analysis of the Proposed Off-Grid Solar PV System

## Project Details

| Parameter | Value |
| --- | --- |
| Project Type | Off-Grid Solar PV System |
| Location | Bilhari, Madhya Pradesh, India |
| Building | Three-Storey Residential Building |
| Daily Energy Demand | 52.5 kWh/day |
| PV Capacity | 15.30 kWp |
| Battery Bank | 48 V, 1664 Ah |
| Battery Type | HERE DEKA 12 V 208 Ah Sealed Gel |
| Battery Quantity | 32 |
| MPPT Controllers | 5 × Victron SmartSolar MPPT 150/100 |
| Project Lifetime | 25 Years |

## Initial Capital Cost

| Component | Quantity | Unit Cost (₹) | Total Cost (₹) |
| --- | --- | --- | --- |
| LG 450 W Solar Modules | 34 | 12,500 | 425,000 |
| HERE DEKA Batteries | 32 | 42,000 | 1,344,000 |
| Victron MPPT 150/100 | 5 | 62,000 | 310,000 |
| 48 V Pure Sine Wave Inverter (15 kVA) | 1 | 300,000 | 300,000 |
| Mounting Structure | 1 Lot | 180,000 | 180,000 |
| DC Combiner Box | 1 | 40,000 | 40,000 |
| AC Distribution Board | 1 | 30,000 | 30,000 |
| Cables & Connectors | 1 Lot | 120,000 | 120,000 |
| Lightning Protection | 1 | 35,000 | 35,000 |
| Earthing System | 1 | 25,000 | 25,000 |
| Installation & Commissioning | 1 Lot | 200,000 | 200,000 |

Total Initial Investment: ₹3,009,000 (≈ ₹30.09 lakh)

## Annual Energy Generation

Assumptions:
• Average Peak Sun Hours = 5.5 h/day
• Performance Ratio = 0.77

E = 15.3 × 5.5 × 365 × 0.77 = 23,640 kWh/year (≈23.6 MWh/year)

## Annual Electricity Cost Savings

Electricity Tariff = ₹8.5/kWh
Annual Savings = 23,640 × 8.5 = ₹200,940/year (≈₹2.01 lakh/year)

## Operation & Maintenance Cost

Annual O&M = 1.5% of capital cost = ₹45,135/year

## Net Annual Savings

₹200,940 − ₹45,135 = ₹155,805/year (≈₹1.56 lakh/year)

## Simple Payback Period

Payback = ₹30.09 lakh / ₹1.558 lakh/year = 19.3 years

## Battery Replacement

Expected Battery Life: 10 years
Replacement Cost: ₹13.44 lakh
Replacement Years: Year 10 and Year 20

## Levelized Cost of Energy (LCOE)

Life-cycle Cost = Initial Investment (₹30.09 lakh) + Battery Replacements (₹26.88 lakh) + O&M (₹11.28 lakh) = ₹68.25 lakh

Lifetime Energy = 23,640 × 25 = 591,000 kWh

LCOE = ₹68.25 lakh / 591,000 kWh = ₹11.55/kWh

## Return on Investment (ROI)

Lifetime Savings = 23,640 × ₹8.5 × 25 = ₹50.24 lakh

ROI = ((50.24 − 30.09) / 30.09) × 100 = 66.97%

## Economic Summary

| Parameter | Value |
| --- | --- |
| Total Investment | ₹30.09 lakh |
| Annual Generation | 23,640 kWh |
| Annual Savings | ₹2.01 lakh |
| Annual O&M | ₹45,135 |
| Net Annual Savings | ₹1.56 lakh |
| Simple Payback | 19.3 Years |
| Battery Replacement | Years 10 & 20 |
| Project Life | 25 Years |
| LCOE | ₹11.55/kWh |
| Lifetime Energy | 591 MWh |
| Estimated ROI | 67% |

## Engineering Discussion

The proposed 15.30 kWp off-grid PV system is technically capable of meeting the 52.5 kWh/day residential load with one day of battery autonomy. Economically, the high upfront investment is primarily due to the large sealed-gel battery bank, which represents nearly 45% of the capital cost. Although the simple payback period is approximately 19 years, the system provides energy independence, protection against grid outages, and reduced exposure to future electricity price increases. If battery prices decrease or lithium iron phosphate (LiFePO₄) batteries are adopted, both the payback period and LCOE would improve significantly.