# Load Calculation Spreadsheet

Project: Off-Grid Solar PV System for Three-Storey Residential Building

| Sl. No. | Floor | Load Description | Qty | Rated Power (W) | Operating Hours (h/day) | Diversity Factor | Connected Load (W) | Daily Energy (Wh/day) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Ground | LED Lights | 4 | 50 | 7 | 1.00 | 200 | 1400 |
| 2 | First | LED Lights | 4 | 50 | 7 | 1.00 | 200 | 1400 |
| 3 | Second | LED Lights | 4 | 50 | 7 | 1.00 | 200 | 1400 |
| 4 | Utility | Water Pump | 2 | 1000 | 1.5 | 1.00 | 2000 | 3000 |
| 5 | Kitchen | Refrigerator | 2 | 200 | 24 | 1.00 | 400 | 9600 |
| 6 | Laundry | Washing Machine | 1 | 1350 | 2 | 1.00 | 1350 | 2700 |
| 7 | Living | Domestic Appliances | 1 | 1200 | 1 | 1.00 | 1200 | 1200 |
| 8 | Bedrooms | Air Conditioner | 2 | 1500 | 6 | 1.00 | 3000 | 18000 |
| 9 | All Floors | Ceiling Fans | 10 | 75 | 12 | 1.00 | 750 | 9000 |
| 10 | Standby | Standby Loads | 1 | 200 | 24 | 1.00 | 200 | 4800 |
|  |  | TOTAL |  |  |  |  | 9500 | 52500 (52.5 kWh/day) |

## Design Summary

| Parameter | Value |
| --- | --- |
| Location | Bilhari, India (23.1261°N, 79.9661°E) |
| Daily Load | 52.5 kWh/day |
| PV Array | 15.30 kWp |
| Battery Bank | 96 V, 643 Ah |
| Battery Type | DEKA Classic 12 V 208Ah Pb Sealed Gel Solar PV 8G8D |
| Accepted PLOL | 5% |